TOWN OF BEAUFORT

CARTERET COUNTY, NORTH CAROLINA

WATER SHORTAGE 
RESPONSE PLAN

June 12th, 2023

WATER SHORTAGE RESPONSE SUMMARY

Recently enacted rules governing water use during droughts and water supply emergencies (15A NCAC 02E .607) require water systems that have to prepare a Local Water Supply Plan to address the following points in their Water Shortage Response Plan.

| A. Who (Name and Title) is responsible for implementing the water shortage response protocols? |  |
| --- | --- |
| Todd Clark Town Manager (252) 728-2141 t.clark@beaufortnc.org | Mr. Donovan H. Willis Public Utilities Director (252) 838-0917 d.willis@beaufortnc.org |
| B. How will water users and system employees be notified when the water shortage response protocols are activated? |  |
| Employee email notification, notices at municipal buildings, Town of Beaufort website, notice in Carteret County News-Times, Public Service Announcement on local radio and television stations, and notices in water bills. |  |
| C. How will water users be informed of required response measures? |  |
| WSRP provisions will be published in Carteret County News-Times and posted on the Town's website. Provisions will be mailed to current customers and new customers. |  |
| D. What specific measurements of available supply, demand, and system conditions will be used to determine the severity of a drinking water shortage? |  |
| The primary condition for determining severity of a drinking water shortage will be the time of Water Treatment Plant operation per day per plant or system failure. |  |
| E. What specific conditions will be used to trigger drinking water use reduction measures and the movement to more restrictive and less restrictive measures? |  |
| Level 1: WTP operation is greater than 12 hours per day per plant. Level 2: WTP operation is greater than 14 hours per day per plant. Level 3: WTP operation is 24 hours per day per plant. |  |
| F. What levels of response are required? |  |
| Level 1: Voluntary Conservation- Conditions indicate potential for shortages; voluntary conservation is encouraged; 5% reduction goal. Education is escalated including fliers and noticed suggesting limiting irrigation to 1 inch per week, preventing water and runoff from impervious services, washing only full loads in washer/dishwasher, eliminating play with sprinklers and hoses, and repairing any known leaks. Level 2: Mandatory Restriction- Water supplies are measurably lower than the seasonal norm and are diminishing; restrictions are imposed; 10% reduction goal from previous month's usage. Encourage continued voluntary actions. Limit irrigation to 1/2" per week between 8PM and 8AM. Ban use of potable water for washing impervious surfaces. Limit testing and training that uses potable water. Level 3: Emergency Response- The system is experiencing water shortage; drinking water supply is inadequate and more stringent restrictions are imposed; 25% reduction goal from previous month's usage. Continue Mandatory Restrictions from Level 2. Ban all non-essential uses of potable water. Limit irrigation to minimum require for necessary plants' survival. |  |
| G. How will the provisions of the water shortage response protocols be enforced? |  |
| An email contact will be set up to report violators. Violations can also be reported to the Public Works Department. Citations will be issued as follows: first a written warning, second a $200 fine and a warning, third a $500 fine, fourth an additional $500 fine, and fifth an additional $500 fine and a service cutoff for five days. For Level 3 emergency response the rates structure will also be increased by 50%. |  |
| H. How will your customers be given opportunities to review and comment on the water shortage response protocols before they are finalized? |  |
| Plan will be made available for public review prior to approval of any revisions by the Town Board. Each time a change is proposed to the Plan it will be published in the Carteret County News-Times and on the Town’s website at least thirty days before the Board prepares to discuss its acceptance. This will allow the public adequate time to review the change and appear at the Board’s meeting to provide input. |  |
| I. How will variance requests be processed? |  |
| Applications will be available from the public works office and the Town's website that can be submitted to the Public Works Director's office. A review will be performed and a decision to grant or deny the variance will be made by the Public Works Director. |  |
| J. What criteria will be used to decide if a variance will be granted? |  |
| Several factors will be considered, including impact on water demand, essential use of drinking water, duration of the variance, alternatives to the variance, social and economic importance, and prevention of structural damage. |  |
| K. How will the effectiveness of the water shortage response protocols be measured? |  |
| Effectiveness of water shortage response protocols will be based upon frequency of activation, attainment of desired reductions upon activation, and number or severity of violation citations. |  |
| L. How will the protocols be revised when needed? |  |
| The Public Works Director and Town Manager will review the implementation and effectiveness of the Water Shortage Response Plan after each activation, at least every five years, or after a change in the system, and will make recommendations to the Town Board. |  |
| M. Does the system depend on water storage in an impoundment not owned by the system? NO |  |
| N. If yes, are the water shortage response protocols consistent with the Water Shortage Response Plan provisions established by the owner of the impoundment? N/A |  |

1.0	INTRODUCTION

Recently adopted rules governing water use during droughts and water emergencies dictate that water systems which are required to submit a Local Water Supply Plan shall also submit a Water Shortage Response Plan (WSRP) for review by the Division of Water Resources.  The requirements are outlined in North Carolina Administrative Code 15A NCAC 02E. 0607.  It is the intent of this document to establish a WSRP for the Town of Beaufort to minimize the adverse effects of drought and water emergencies while satisfying the requirements of the State.

2.0	BACKGROUND

The Town of Beaufort is located within Carteret County, North Carolina, situated on a peninsula bounded generally by the North River, Taylor’s Creek, and the Newport River, and lies within the White Oak River Basin.

Beaufort’s designated utility service area includes all of the Town limits, its approximate one-mile extraterritorial area, plus approximately 4.5 additional square miles of land.

The existing public water system serves approximately 4,144 individual water accounts.  All but 12 of these accounts are located within the existing corporate limits.  A breakdown of the accounts shows residential customers are 86%, commercial customers are 8%, sprinklers are 2.7%, institutional customers are 1.3%, docks are 1.2%, and governmental services are 0.4%.

The Town of Beaufort’s current water supply is groundwater taken from the Castle Hayne aquifer.  The water quality for the existing Castle Hayne wells requires treatment in order to meet Potable Drinking Water Standards.  Raw water is pumped from the aquifer by four wells to the two existing water treatment plants.  The Pine Street WTP is the older of the two treatment facilities.  It is in excess of 30 years old and is located within the Public Works Complex.  The Glenda Drive WTP is in excess of 30 years old.  Both of these WTPs provide treatment via ion exchange softeners with brine utilized for recharge.  Cascade tray aerators provide removal of hydrogen sulfide while disinfection is achieved via chlorine gas injection.  On-site storage is provided via a ground storage tank at each plant site.

Treated water is stored in the ground storage tanks prior to being pumped into the distribution system and elevated tanks.  The Town currently has two elevated storage tanks that provide approximately 400,000 gallons of storage.  The elevated storage tanks provide sufficient storage capacity to maintain system pressures.

3.0	RESPONSIBLE PARTIES

Two individuals are responsible for implementing the water shortage protocols:

Mr. Todd Clark

Town Manager

(252) 728-2141

t.clark@beaufortnc.org

Mr. Donovan H. Willis

Public Utilities Director

(252) 838-0197

d.willis@beaufortnc.org

If either of these individuals no longer holds his or her position then the responsibilities entailed here carry to their successor.  In the event that neither individual can fulfill their duties in implementing the water shortage protocols the Mayor and Town Board will be responsible for implementing the protocol.

4.0	ACTIVATION NOTIFICATION METHODS

It is vital to the effectiveness of this plan that water users and system employees are immediately informed when water shortage response protocols are activated.  Upon activation, employee emails will be sent out and notices at municipal buildings will be posted to inform system employees of the protocols.  Water users will be notified of water shortage response activation by a posting to the Town of Beaufort website, a notice in the Carteret County News-Times, notices in water bills, and a public service announcement on local radio and television stations.

5.0	NOTIFICATION OF REQUIRED RESPONSE MEASURES

Water Shortage Response Plan provisions will be published in the Carteret County News-Times annually and will be available on the Town’s website.  Provisions will be mailed annually to all current customers and will be provided to new customers at the time of account activation.  In addition, if changes to the Plan are made, the changes will be published in the News-Times and notification will be provided to current customers.

6.0	SEVERITY CRITERIA

The primary condition for determining the severity of a drinking water shortage will be the length of time (hours) that each Water Treatment Plant must operate per day in order for supply to meet the required demand.  In addition, severity can be affected as deemed appropriate by the Town Manager or Public Utilities Director by mechanical failure of the water treatment and distribution systems.

7.0	TRIGGER CONDITIONS

There will be three levels of response, triggered by the aforementioned conditions of severity:

- Level 1 will be activated if Water Treatment Plant Operation is greater than 12 hours per day per plant or if the Town Manager or Public Utilities Director deems it appropriate based upon other unanticipated operating conditions.

- Level 2 will be activated if Water Treatment Plant Operation is greater than 14 hours per day per plant or if the Town Manager or Public Utilities Director deems it appropriate based upon other unanticipated operating conditions.

- Level 3 will be activated if Water Treatment Plant Operation is 24 hours per day per plant or if the Town Manager or Public Utilities Director deems it appropriate based upon other unanticipated operating conditions.

8.0	LEVELS OF RESPONSE

Three levels of response have been selected, each corresponding to various conditions set forth above.  Each level of response has a unique set of goals and strategies as shown here:

- Level 1—Voluntary Conservation:  A 5% reduction goal from the previous month for water consumption is set.  Conditions indicate that there is a potential for shortages.  Voluntary conservation is encouraged.  Education efforts are ramped up to encourage water conservation, and any evidence of leaks is investigated and corrected as seen fit.  Education efforts should suggest limiting irrigation to one inch per week, preventing water and runoff from impervious services, washing only full loads in the washer or dishwasher, eliminating play with sprinklers and hoses, and repairing any known leaks.

- Level 2—Mandatory Restriction:  A 10% reduction goal from the previous month for water consumption is set.  Water supplies are measurably lower than the seasonal norm and are diminishing.  Encourage continuation of all voluntary reductions.  Restrictions on non-essential uses of water are imposed.  Irrigation is limited to one half inch per week between 8PM and 8AM.  Use of potable water for washing impervious services is banned.

- Level 3—Emergency Response:  A 25% reduction goal from the previous month for water consumption is set.  The system is experiencing water shortage at this level and drinking water supply is inadequate.  More stringent restrictions are imposed.  Continue all restrictions from Level 2.  No non-essential uses of water are allowed and other uses of water may be restricted if deemed necessary by the Public Works Director and Town Manager.  Irrigation is limited strictly to minimum required amount for the survival of necessary plants.

For the purposes of this Plan, non-essential uses of drinking water are as follows:

- Any ornamental use such as fountains, reflecting pools, and artificial water falls

- Watering gardens, lawns, parks, golf courses (except greens), playing fields, and other recreational areas

- Filling and operation of recreational swimming pools which serve fewer than 25 dwellings

- Non-commercial washing of motor vehicles

- Serving water in restaurants, clubs, or eating places except by specific request

- Refilling of air conditioning cooling towers, except for those used by health care facilities or domestic use if it is deemed necessary to sustain standards of hygiene, health, and sanitation

9.0	ENFORCEMENT

A hotline and an email contact will be set up so that citizens can report violators of the restrictions set after activation of the Plan.  In addition, a citation system will be set up to be administered by the Town Manager as follows:

- A first offense will warrant a written warning.

- Second offenses will result in a $200 fine and a warning.

- Third offenses will result in a $500 fine.

- Fourth offenses will result in an additional $500 fine.

- Fifth offenses will result in an additional $500 fine and a five day service cutoff.

In addition, during activation of Level 3 Emergency Response protocols the rates for water consumption will be increased by 50% to further discourage water waste and to compensate for lost revenues of the water system.

10.0	USER REVIEW

Understanding that review by water users is essential to cooperation and understanding, this Plan will be made available for public review prior to approval of any revisions by the Town Board.  Before initial implementation and each time a change is proposed to the Plan it will be published in the Carteret County News-Times and on the Town’s website at least thirty days before the Board prepares to discuss its acceptance.  This will allow the public adequate time to review the change and appear at the Board’s meeting to provide input.

11.0	VARIANCES

Applications for variances from the Water Shortage Response Plan’s protocols will be made available from the public works office and printable from the Town’s website.  These applications will be submitted to the Public Utilities Director’s office.  The Public Utilities Director will perform a review and make a decision to grant or deny the variance.  In reviewing the request, the Public Utilities Director will consider impact on water demand, essential use of drinking water, duration of the variance, alternatives to the variance, social and economic importance, prevention of structural damage, and any other factors deemed important.

12.0	EVALUATION OF EFFECTIVENESS & REVISION

Effectiveness of the Water Shortage Response Plan will be reviewed jointly by the Public Utilities Director and the Town Manager after each activation of the protocol, or at a maximum of a five year interval.  In addition, the Plan must be reviewed to adapt to system additions, alterations, or changes in general circumstance.  Recommendations will be made as deemed appropriate to the Town Board.  Effectiveness will be measured based upon numerous subjective and objective factors but should include frequency of activation, attainment of desired reductions upon activation, and number or severity of violation citations.  As daily measurements of run times at each treatment plant are already recorded, quantifying water use reduction will be practically instantaneous and will be the primary method for establishing the Plan’s effectiveness.

Todd Clark				 Town Manager

t.clark@beaufortnc.org

Phone 252-728-2141

Donovan H. Wills                   Public Utilities Director

d.willis@beaufortnc.org

Phone 252-838-0197

Wade Keeler                 		Operator in Responsible Charge

w.keeler@beaufortnc.org

Phone 252-808-5425