Reviewed & Updated 5/23/23

The procedures herein are written to reduce potable water demand and supplement existing drinking water supplies whenever they are in danger of being inadequate to meet customer needs.

I. Authorization

The city manager for the City of Morganton is authorized to enact water shortage response provisions when ever the trigger conditions outlined in Section IV are met.  In her absence, the Water Resources Director will assume this role.

Ms. Sally Sandy				Mr. Brad Boris

Morganton City Manager			Water Resources Director

Phone: 828-438-5230				Phone: 828-438-5285

Email: citymanager@ci.morganton.nc.us	email: bboris@ci.morganton.nc.us

The Water Shortage Management Plan will be initiated for one of two reasons. One could be a failure in Morganton’s Water System such as a major line break or facility failure at the water plant or major pump station causing significant water loss for Morganton water customers or as response to a regional drought.  A regional drought will be identified by the State of North Carolina DENR’s Division of Water Resources or Duke Energy of the Carolinas as they enact and declare a drought and call the Catawba Wateree Drought Management Advisory Group to respond to a drought trigger as outlined in Section IV of this document.  According to Section 5-149(d) the Water Resources Director is authorized to adopt and issue a water shortage management plan with the approval of the City Manager.  Once triggers have been promulgated, the City will follow trigger level actions as outlined below.

II. Notification

The following notification methods will be used to inform water system employees and customers of a water shortage declaration: employee email announcements, notices at municipal buildings, and notices in water bills. Required water shortage response measures will be communicated through PSA announcements on local radio and cable stations, and on the City’s website (http:/www.ci.morganton.nc.us/).  Declaration of emergency water restrictions or water rationing will be communicated to all customers by telephone with reverse 911.

III. Levels of Response

Five levels of water shortage response are outlined below.  The five levels of water shortage response are voluntary reductions, mandatory reductions I and II, emergency reductions, and water rationing. A detailed description of each response level and corresponding water reduction measures follow below.

| Stage | Response | Description |
| --- | --- | --- |
| 1 | Voluntary Reductions | Water Users are encouraged to reduce their water use and improve water use efficiency; however, no penalties apply for noncompliance. Water supply conditions indicate a potential for shortage. |
| 2 | Mandatory Reductions I | Water users must abide required water use reduction and efficiency measures; penalties apply for noncompliance. Water supply conditions are significantly lower than the seasonal norm and water shortage conditions are expected to persist. |
| 3 | Mandatory Reductions II | Same as Stage 2 |
| 4 | Emergency Reductions | Water supply conditions are subsequently diminished and pose an imminent threat to human health or environmental integrity. |
| 5 | Water Rationing | Water supply conditions are subsequently diminished and remaining supplies must be allocated to preserve human health and environmental integrity. |

In Stage 1, Voluntary Reductions, all water users will be asked to reduce their normal water use by 5%. Customer education and outreach programs will encourage water conservation and efficiency measures including: irrigating landscapes a maximum of one inch per week; preventing water waste, runoff and watering impervious surfaces; watering plants deeply to encourage root growth; washing only full loads in clothes and dishwashers; using spring-loaded nozzles on garden hoses; and identifying and repairing all water leaks.

In Stage 2, Mandatory Reductions I, all customers are expected to reduce their water use by 10% in comparison to their previous month’s water bill. In addition to continuing to encourage all voluntary reduction actions, the following restrictions apply: irrigation is limited to a half inch per week between 8PM and 8AM; outdoor use of drinking water for washing impervious surfaces is prohibited; and all testing and training purposes requiring drinking water (e.g. fire protection) will be limited.

In Stage 3, Mandatory Reductions II, customers must continue actions from all previous stages and further reduce water use by 20% compared to their previous month’s water bill. All non-essential uses of drinking water are banned and garden/landscape irrigation must be reduced to the minimum amount necessary for survival.

In Stage 4, Emergency Reductions, customers must continue all actions from previous stages and further reduce their water use by 25% compared to their previous month’s water bill. A ban on all use of drinking water except to protect public health and safety is implemented.

The goal of Stage 5, Water Rationing, is to provide drinking water to protect public health (e.g. residences, residential health care facilities, and correctional facilities). In Stage 5, all customers are only permitted to use water at the minimum required for public health protection. Firefighting is the only allowable outdoor water use and pickup locations for distributing potable water will be announced according to Morganton’s Emergency Response Plan.

IV. Triggers

Morganton is a signer of the Catawba Wateree Low Inflow Protocol (LIP) and therefore will abide with their trigger points. When CW declares a water shortage, the City of Morganton will as well.

Return to Normal

When water shortage conditions have abated and the situation is returning to normal, water conservation measures employed during each phase should be decreased in reverse order of implementation. Permanent measures directed toward long-term monitoring and conservation should be implemented or continued so that the community will be in a better position to prevent shortages and respond to recurring water shortage conditions.

V. Enforcement

The provisions of the water shortage response plan will be enforced by the City of Morganton Water Resources Department and police personnel. Violators may be reported to the City’s dedicated water conservation hotline. Citations are assessed according to the following schedule depending on the number of prior violations and current level of water shortage.

| Water Shortage Level | First Violation | Second Violation | Third Violation |
| --- | --- | --- | --- |
| Voluntary Reductions | N/A | N/A | N/A |
| Mandatory Reductions (Stages 2 and 3) | $50 | $100 | $200 |
| Emergency Reductions | $150 | $300 | $600 |
| Water Rationing | $500 | Discontinuation of Service | Discontinuation of Service |

VI. Public Comment

Customers will have multiple opportunities to comment on the provisions of the water shortage response plan. First, a draft plan will be available at City Hall for customers to view. A notice will be included in customer water bills notifying them of such. Also, a draft plan will be published on the city’s website. All subsequent revisions to the draft plan will be published at least 30 days prior to an adoption vote by Morganton’s City Council.

VII. Variance Protocols

Applications for water use variance requests are available from the Morganton Water Resources Office. All applications must be submitted to the Water Resources office for review by the Water Resources Director or his or her designee.  A decision to approve or deny individual variance requests will be determined with in two weeks of submittal after careful consideration of the following criteria: impact on water demand, expected duration, alternative source options, social and economic importance, purpose (i.e. necessary use of drinking water) and the prevention of structural damage.

VIII. Effectiveness

The effectiveness of the City of Morganton water shortage response plan will be determined by comparing the stated water conservation goals with observed water use reduction data. Other factors to be considered include frequency of plan activation, any problem periods without activation, total number of violation citations, desired reductions attained and evaluation of demand reductions compared to the previous year’s seasonal data.

IX. Revision

The water shortage response plan will be reviewed and revised as needed to adapt to new circumstances affecting water supply and demand, following implementation of emergency restrictions, and at a minimum of every five years in conjunction with the updating of our Local Water Supply Plan. Further, a water shortage response planning work group will review procedures following each emergency or rationing stage to recommend any necessary improvements to the plan to Morganton’s City Council.

The City of Morganton Water Resources Director is responsible for initiating all subsequent revisions.