Town of Fallston Water Shortage Response Plan

Fallston, North Carolina

August 17, 2023

The procedures herein are written to reduce potable water demand and supplement existing drinking water supplies whenever they are in danger of being inadequate to meet customer needs.

I. Authorization

The Town of Fallston Mayor is authorized to enact water shortage response provisions whenever initiated by Cleveland County Water. In his or her absence, the Town Clerk will assume this role.

Fallston Mayor: Doris Weaver

Phone: (704) 538-7652

Ms. Mary Boggs

Fallston Town Clerk

Phone: (704) 538-7176

E-mail: townoffallstonnc@bellsouth.net

II. Notification

The following notification methods will be used to inform water system employees and customers of a water shortage declaration: notices at municipal buildings and notices in water bills. Required water shortage response measures will be communicated through The Shelby Star and notices in water bills. Declaration of emergency water restrictions or water rationing will be communicated to all customers by The Shelby Star and/or door hangers.

III. Levels of Response

Five levels of water shortage response are outlined in the table below. The five levels of water shortage response are: voluntary reductions, mandatory reductions I and II, emergency reductions and water rationing. A detailed description of each response level and corresponding water reduction measures follow below.

| Stage | Response | Description |
| --- | --- | --- |
| 1 | Voluntary Reductions | Water users are encouraged to reduce their water use and improve water use efficiency; however, no penalties apply for noncompliance. Water supply conditions indicate a potential for shortage. |
| 2 | Mandatory Reductions I | Water users must abide required water use reduction and efficiency measures; penalties apply for noncompliance. Water supply conditions are significantly lower than the seasonal norm and water shortage conditions are expected to persist. |
| 3 | Mandatory Reductions II | Same as in Stage 2 |
| 4 | Emergency Reductions | Water supply conditions are substantially diminished and pose an imminent threat to human health or environmental integrity. |
| 5 | Water Rationing | Water supply conditions are substantially diminished and remaining supplies must be allocated to preserve human health and environmental integrity. |

STAGE I VOLUNTARY REDUCTIONS

The goal for stage I will be a 5 % reduction in water demand. Stage I will be initiated when required by Cleveland County Water. The following should be encouraged under Stage I Response:

- An educational campaign will be initiated to inform the public of the voluntary water restrictions.

- Residential customers will be encouraged to practice the following conservational measures:

a. Use shower for bathing rather than a bathtub whenever possible. Try and limit

the time to 5 minutes.

b. Do not leave water running while shaving or washing dishes

c. Run dish washers and clothes washers only when fully loaded.

d. Do not use sprinklers just to cool off or for play.

e. Delay new landscaping and seeding until a later date whenever possible.

f. Limit existing lawn watering to that necessary for plants to survive.

g. Water shrubbery the minimum required, reusing household water when possible.

h. Limit car washing to a minimum.

i. Limit washing down of outside areas such as sidewalks, patios, siding, etc.

j. Install water saving devices in shower heads whenever possible.

k. Install water saving devices in toilets such as water filled plastic jugs.

l. Use disposable dishes and utensils when possible.

Stage II Mandatory Reductions I

The Goal for Stage II will be a 10% reduction in water demand. This is the second stage of drought conditions. Stage II will be initiated when required by Cleveland County Water. If the conditions for a Stage II reduction are met, the following mandatory restrictions would be added in addition to the voluntary water recommendations from a Stage I drought:

It shall be unlawful to:

- Water lawns, grass, shrubbery, trees, and flower and vegetable gardens except as follows:

- Such watering may only be done between the hours of 8PM and 8AM.

- Watering by a sprinkler system is limited to a rate of ½ inch per week; or customers may water by hand-held hose, with a container or with a drip irrigation system.

Provided, however, that any person regularly engaged in the sale of plants shall be permitted to use a minimal amount of water at any time to keep plants living.

- Introduce water into any ornamental fountain pool or pond or make similar use of water except to keep fish alive in an existing pond.

3. Wash automobiles, trucks, trailers, boats, campers, or any other type of mobile equipment including commercial washing establishments unless 50% of the water is recycled. The equipment may be washed if it cannot be safely operated unless cleaned.

4. Operate water-cooled air conditioners or other equipment that does not recycle cooling water, except when health and safety are adversely affected.

5. Fill newly constructed swimming and/or wading pools, which have been drained. A minimal amount of water may be added to maintain continued operation of an existing pool.

6. Wash down outside areas such as streets, driveways, parking lots, office buildings, exteriors of existing or newly constructed homes or apartments, sidewalks or patios, or use water for other similar purposes unless done so for public safety or sanitation.

7. Use water from public or private fire hydrants for any purpose other than fire suppression or other public emergency or by Town Water Dept staff.

8. Use water for dust control or compaction.

9. Use potable water for fire department training or equipment testing unless required by State or Federal regulations.

10. Use of water for any unnecessary purpose or intentionally waste water.

STAGE III MANDATORY REDUCTIONS II

The goal for Stage III will be a 15% reduction in water demand. Stage III will be initiated when required by Cleveland County Water. In addition to the restrictions set forth in Stage I and Stage II, the following additional water restrictions shall be imposed.

1. Contractors are asked to reduce water usage to essential needs.

It shall be unlawful to:

1. Water lawns, grass, shrubbery, trees, and flowers vegetable gardens except as follows:

A. Such watering may be done on Saturday only from 8PM to Sunday 8AM.

B. Such watering is to be done by hand-held hose, by container, or by drip irrigation system.

Provided, however, that any person regularly engaged in the sale of plants shall be permitted to use a minimal amount of water at any time to keep plants living.

2. Add any water from Town water supply to an ornamental pond.

3.  Wash automobiles, trucks, trailers, boats, campers, or any other type of mobile equipment including commercial washing establishments unless 75% of the water is recycled. The equipment may be washed if it cannot be safely operated unless cleaned.

- Refuse to repair water leaks on property or facilities of Town of Fallston customers within (10) days of discovery and notification by the Town of Fallston.

STAGE IV EMERGENCY REDUCTIONS

The goal for Stage IV will be a 25% reduction in water demand. Stage IV will be initiated when required by Cleveland County Water. In addition to the restrictions set forth in Stages I-III the following restrictions shall be imposed:

It shall be unlawful to:

1. Use water outside a structure for any use other than an emergency, a fire, for public safety, of use by Town Water Dept. to maintain the system. This includes car washes unless 100% of the water is recycled, any watering of lawns and shrubs, and outside use by a contractor unless to finish a portion of a job.

2. Operate an evaporative air conditioning unit that recycles water except during the operating hours of the business. An exception is made when health and safety are adversely affected.

3. Refuse to repair water leaks within 10 days of notification. Large leaks will be immediately shut off from service.

4. Large Industrial customers are asked to delay projects consuming large quantities of water where possible.

STAGE V WATER RATIONING

The goal for Stage V (Water Rationing) is to provide drinking water to protect public health. This will be initiated when requested by Cleveland County Water. In addition to the restrictions imposed in Stages I-IV, the following restrictions shall be imposed:

1. A system of water rationing shall be put in effect. In the event of water rationing, water will be supplied in the minimal quantities required for the health, welfare, and safety of the citizens in accordance with a program determined by the Fallston Town Council.

2. It shall be unlawful to fail to act in accordance therewith or to use water in any manner or attempt to evade or avoid such water rationing restrictions.

3. Fire protection will be maintained, but where possible, tank trucks shall use raw water.

At any time the Town of Fallston will implement the response stage that corresponds to Cleveland County Water and the Water Shortage Response Plan of Cleveland County Water.

IV. Enforcement

The provisions of the water shortage response plan will be enforced by Town of Fallston Town Clerk. Violators may be reported to the Town office. Citations are assessed according to the following schedule depending on the number of prior violations and current level of water shortage.

| Water Shortage Level | First Violation | Second Violation | Third Violation |
| --- | --- | --- | --- |
| Voluntary Reductions | Warning | $50 | $100 |
| Mandatory Reductions (Stages 2 and 3) | Warning | $100 | Discontinuation of Service |
| Emergency Reductions | $250 | Discontinuation of Service | Discontinuation of Service |
| Water Rationing | $500 | Discontinuation of Service | Discontinuation of Service |

V. Public Comment

Customers will have multiple opportunities to comment on the provisions of the water shortage response plan. First, a draft plan will be available at the Fallston Town Office, P.O Box 547 Fallston. NC. A public hearing will be scheduled at the Regular Town Board Meeting so that comments may be made. All subsequent revisions to the draft plan will be published at least 30 days prior to an adoption vote by the Fallston Town Council.

VI. Variance Protocols

Applications for water use variance requests are available from the Town of Fallston Town Office.  All applications must be submitted to the Town Office for review by the Town Clerk or his or her designee. A decision to approve or deny individual variance requests will be determined within two weeks of submittal after careful consideration of

the following criteria: impact on water demand, expected duration, alternative source options, social and economic importance, purpose (i.e. necessary use of drinking water) and the prevention of structural damage.

VII. Effectiveness

The effectiveness of the Town of Fallston water shortage response plan will be determined by comparing the stated water conservation goals with observed water use reduction data. Other factors to be considered include frequency of plan activation, any problem periods without activation, total number of violation citations, desired reductions attained and evaluation of demand reductions compared to the previous year’s seasonal data.

VIII. Revision

The water shortage response plan will be reviewed and revised as needed to adapt to new circumstances affecting water supply and demand, following implementation of emergency restrictions, and at a minimum of every five years, as required by the provisions of G.S. 143-355 (l). Further, a water shortage response planning work group will review procedures following each emergency or rationing stage to recommend any necessary improvements to the plan to Fallston Town Council. The Town of Fallston Town Clerk is responsible for initiating all subsequent revisions.